# PARCHAM · Serial Dhwani — Intercept GC-0914

Write-up: recovering the Ghost Carrier flag (915 MHz ISM, SigMF ci16_le, 1 MHz, 4 s)

## Flag

RFCTF{ph4nt0m_sp3ctrum_b3n34th_th3_fl00r_0f_n01s3}

Verified against the manifest SHA-256. The manifest line was truncated by one missing packet, but all 59 hex characters available match the flag’s hash: 40098199b78bfdd100d45a68631dd4014638a63ad73bffccb557d37cefd… (full hash …cefdccb7a).

## Stage 1: the hopping packet network

- Channels. 16 hop channels at about 49.95 kHz spacing, from −375 kHz to +374 kHz. Bursts last 9.55 ms. Two strong narrowband interferers near ±455 kHz are unrelated to the hopper.

- Modulation. 2-FSK at 25 kBd with roughly ±6 kHz deviation; per-burst centre frequency wanders by 1–3 kHz.

- Frame start. Each burst has an alternating preamble and a fixed 32-bit sync word. Brute force gave bit offset 66, normal polarity.

- Decoding. De-whitened with the per-channel seed; CRC-16/CCITT-FALSE passed. Every frame has total=23, len=16.

- Packets. Packets 0–21 decoded and all duplicate copies agree. Packet 22 was in a damaged or fragmented burst and was not recovered.

- Payload. Begins 78 da (zlib) and decompresses to the PHANTOM/1 relay manifest: DSSS/DBPSK, GPS C/A codes at 125 kchip/s, key split into shares, plus the ciphertext and decryption rule. Packet 22 held only the manifest tail, hence the truncated SHA-256 line.

## Stage 2: the ghost carrier

- Acquisition. Despreading at 8 samples/chip, 1023-chip period. The signals sit below the noise floor, so the ±455 kHz interferers were notched and PRN × carrier offset × code phase was searched. PRNs 17, 19 and 32 stood out.

- Carrier and timing. Each carrier is a linear chirp with its own chip-clock offset: −141.9 Hz/s (PRN 17), −147.2 Hz/s (PRN 19), +66.2 Hz/s (PRN 32). A fixed-frequency correlation therefore looked smeared.

- Hop-burst interference. Blanking the hop bursts zeroed whole code periods and broke the CRC. Notching only the narrowband hop channel preserved the wideband DSSS energy.

- DBPSK demodulation. With chirp tracking, differential phase coherence was 0.97 on all three carriers (one bit per 8.184 ms code period). Searching for the F0C3 magic yielded three frames that passed CRC-8 (poly 0x07, init 0):

| PRN | idx | Share (24 bytes) |
| --- | --- | --- |
| 19 | 0/3 | d543865df58cc0b3154bbe2145093a4c8392aa9e0a7219c5 |
| 32 | 1/3 | b452b63916c644e019c61a0fa4e832c9a08a4fbc8234b304 |
| 17 | 2/3 | 5e2a2ddf4dab2a9cc694374f999582d73f468baf2d52203f |

- Key and decryption. XOR of the three shares gives the key; then flag = ct XOR SHAKE256(key)[0:50].

Key: 3f3b1dbbaee1aecfca19936178748a521c5e6e8da5148afe

## Caveat

Hop packet 22 was not recovered. This does not affect the flag: all shares and the ciphertext are intact and the hash matches.